#!/usr/bin/env python3

# -*- coding: utf-8 -*-

"""

龙芯2K1000LA 水果在线测量与分级 —— PyQt5 GUI 展示界面

适配ESP32校准后数据：W = D0 - L - R，H = H0 - T

依赖：PyQt5, pyqtgraph, numpy, pyserial

"""

import sys

import os

import time

import json

import serial

import numpy as np

from PyQt5.QtCore import Qt, QObject, QThread, pyqtSignal, QTimer

from PyQt5.QtWidgets import (QApplication, QMainWindow, QWidget, QVBoxLayout,

QHBoxLayout, QGridLayout, QLabel, QPushButton,

QTableWidget, QTableWidgetItem, QHeaderView,

QSplitter, QStatusBar, QMessageBox, QDialog,

QDialogButtonBox)

import pyqtgraph as pg

# ======================= 系统配置（D0, H0 需与硬件安装值一致） =======================

CONFIG = {

"port": "/dev/ttyS1",

"baudrate": 115200,

# D0: 左右传感器出光点之间的实际距离（毫米），需与ESP32端一致

"D0": 170.0,

# H0: 顶部传感器出光点到传送带表面的实际距离（毫米），需与ESP32端一致

"H0": 110.0,

# ESP32端已经完成 L/R/T 线性校准和平滑，龙芯端不要再次校准

"esp32_data_calibrated": True,

"speed": 20.0,

"enter_threshold": 25.0,

"exit_threshold": 15.0,

"enter_count": 3,

"exit_count": 6,

"fruit_min_frames": 10,

"filter_window": 5,

"cv_ideal": 0.12,

"cv_tolerance": 0.15,

"symmetry_weight": 0.65,

"cv_weight": 0.35,

"grade_rules": {

"A": {"w_min": 70, "w_max": 85, "h_min": 70, "h_max": 85, "r_min": 0.8},

"B": {"w_min": 60, "w_max": 90, "h_min": 60, "h_max": 90, "r_min": 0.6},

"C": {"w_min": 50, "w_max": 100, "h_min": 50, "h_max": 100, "r_min": 0.4}

},

"tmp_dir": "fruit_tmp",

"result_file": "results.jsonl",

}

class FruitProcessor(QObject):

"""水果测量核心逻辑"""

frame_signal = pyqtSignal(float, float, float, float, float)  # L, R, T, w, h

fruit_done_signal = pyqtSignal(dict)

status_signal = pyqtSignal(str)

def __init__(self, config):

super().__init__()

self.cfg = config

self.ser = None

self.in_fruit = False

self.enter_cnt = 0

self.exit_cnt = 0

self.fruit_id = 0

self.w_buffer = []

self.current_tmp_fp = None

self.current_tmp_path = None

self.last_time = None

self._stop_flag = False

os.makedirs(config["tmp_dir"], exist_ok=True)

self._clean_orphan_tmp()

def _clean_orphan_tmp(self):

tmp_dir = self.cfg["tmp_dir"]

if os.path.exists(tmp_dir):

for f in os.listdir(tmp_dir):

if f.endswith('.tmp'):

try:

os.remove(os.path.join(tmp_dir, f))

except:

pass

def open_serial(self):

try:

self.ser = serial.Serial(self.cfg["port"], self.cfg["baudrate"], timeout=0.5)

self.status_signal.emit(f"串口 {self.cfg['port']} 打开成功")

return True

except Exception as e:

self.status_signal.emit(f"串口打开失败: {e}")

return False

def stop(self):

self._stop_flag = True

def parse_line(self, line):

"""

解析ESP32发送的数据。

当前ESP32端发送格式：FRUIT,L=64.2,R=50.1,T=80.3#

这里的 L/R/T 已经是ESP32校准后的距离值，龙芯端直接使用。

"""

try:

line = line.strip()

if not line:

return None

if "IDLE" in line:

return None

# 兼容中转端可能带上的调试前缀，例如：Received: FRUIT,...

if "FRUIT," in line:

data = line.split("FRUIT,", 1)[1]

elif "DATA," in line:

data = line.split("DATA,", 1)[1]

else:

return None

data = data.replace("#", "")

kv = {}

for part in data.split(","):

if "=" in part:

k, v = part.split("=", 1)

kv[k.strip()] = float(v.strip())

if not all(k in kv for k in ("L", "R", "T")):

return None

return kv["L"], kv["R"], kv["T"]

except Exception:

return None

def filter(self, value, buffer):

buffer.append(value)

if len(buffer) > self.cfg["filter_window"]:

buffer.pop(0)

return sum(buffer) / len(buffer)

def open_tmp(self, L, R, T):

self.fruit_id += 1

name = f"fruit_{self.fruit_id}_{int(time.time())}.tmp"

self.current_tmp_path = os.path.join(self.cfg["tmp_dir"], name)

self.current_tmp_fp = open(self.current_tmp_path, "w")

self.last_time = time.time()

self.write_tmp(L, R, T)

self.status_signal.emit(f"进入水果 #{self.fruit_id}")

def write_tmp(self, L, R, T):

if self.current_tmp_fp:

t = time.time()

self.current_tmp_fp.write(f"{t},{L},{R},{T}\n")

self.current_tmp_fp.flush()

def close_tmp(self):

if self.current_tmp_fp:

self.current_tmp_fp.close()

self.current_tmp_fp = None

return self.current_tmp_path

@staticmethod

def _smooth_sequence(seq, window):

if window <= 1 or len(seq) < window:

return seq

smoothed = np.convolve(seq, np.ones(window)/window, mode='same')

return smoothed.tolist()

def compute_roundness(self, W):

if len(W) < 8:

return 0.0

arr = np.array(W)

peak = np.argmax(arr)

front = arr[:peak + 1]

back = arr[peak:][::-1]

n = min(len(front), len(back))

if n < 3:

return 0.0

f_interp = np.interp(np.linspace(0, 1, n),

np.linspace(0, 1, len(front)), front)

b_interp = np.interp(np.linspace(0, 1, n),

np.linspace(0, 1, len(back)), back)

corr = np.corrcoef(f_interp, b_interp)[0, 1]

if np.isnan(corr):

corr = 0.0

cv = np.std(arr) / (np.mean(arr) + 1e-6)

cv_score = max(0.0, 1.0 - abs(cv - self.cfg["cv_ideal"]) / self.cfg["cv_tolerance"])

roundness = (self.cfg["symmetry_weight"] * max(0, corr) +

self.cfg["cv_weight"] * cv_score)

return min(1.0, roundness)

def grade(self, w, h, r):

candidates = []

for grade_name, rule in self.cfg["grade_rules"].items():

if not (rule["w_min"] <= w <= rule["w_max"] and

rule["h_min"] <= h <= rule["h_max"]):

continue

w_mid = (rule["w_min"] + rule["w_max"]) / 2.0

h_mid = (rule["h_min"] + rule["h_max"]) / 2.0

size_score = 1.0 - (abs(w - w_mid) / (rule["w_max"] - rule["w_min"] + 1e-6) +

abs(h - h_mid) / (rule["h_max"] - rule["h_min"] + 1e-6)) / 2.0

size_score = max(0.0, min(1.0, size_score))

final_score = 0.6 * size_score + 0.4 * r

candidates.append((final_score, grade_name))

if not candidates:

return "Reject"

best = max(candidates, key=lambda x: x[0])

if best[0] < 0.5:

return "Reject"

return best[1]

def process(self, path):

"""处理临时文件，生成结果"""

t_list, L_list, R_list, T_list = [], [], [], []

try:

with open(path, "r") as f:

for line in f:

t, L, R, T = line.strip().split(",")

t_list.append(float(t))

L_list.append(float(L))

R_list.append(float(R))

T_list.append(float(T))

except Exception as e:

self.status_signal.emit(f"读取临时文件失败: {e}")

return

if len(L_list) < self.cfg["fruit_min_frames"]:

self.status_signal.emit(f"水果帧数过少 ({len(L_list)})，忽略")

return

D0 = self.cfg["D0"]

H0 = self.cfg["H0"]

W, H = [], []

length = 0.0

for i in range(len(L_list)):

# 修正：宽度 = D0 - L - R，接收的L,R是校准后的距离值

w_raw = D0 - L_list[i] - R_list[i]

h_raw = H0 - T_list[i]

w = max(0.0, w_raw)

h = max(0.0, h_raw)

W.append(w)

H.append(h)

if i > 0:

dt = t_list[i] - t_list[i - 1]

dt = max(0.0, min(dt, 0.1))

length += self.cfg["speed"] * dt

W_smooth = self._smooth_sequence(W, self.cfg["filter_window"])

H_smooth = self._smooth_sequence(H, self.cfg["filter_window"])

max_w = max(W_smooth) if W_smooth else 0.0

max_h = max(H_smooth) if H_smooth else 0.0

roundness = self.compute_roundness(W_smooth)

grade_label = self.grade(max_w, max_h, roundness)

record = {

"id": self.fruit_id,

"time": time.strftime("%Y-%m-%d %H:%M:%S"),

"max_w": round(max_w, 1),

"max_h": round(max_h, 1),

"length": round(length, 1),

"roundness": round(roundness, 3),

"grade": grade_label,

"W": [round(x, 1) for x in W_smooth],

"H": [round(y, 1) for y in H_smooth],

"raw_L": [round(x, 1) for x in L_list],

"raw_R": [round(x, 1) for x in R_list],

"raw_T": [round(x, 1) for x in T_list]

}

with open(self.cfg["result_file"], "a", encoding='utf-8') as f:

f.write(json.dumps(record, ensure_ascii=False) + "\n")

self.fruit_done_signal.emit(record)

self.status_signal.emit(f"水果 #{self.fruit_id} 处理完成，等级: {grade_label}")

def run(self):

if not self.open_serial():

self.status_signal.emit("串口初始化失败，退出")

return

self.status_signal.emit("开始测量...")

try:

while not self._stop_flag:

line = self.ser.readline().decode(errors="ignore")

data = self.parse_line(line)

if data is None:

continue

L, R, T = data

# L/R/T来自ESP32端，已经完成传感器线性校准和平滑

# 龙芯端只负责几何计算：宽度 = D0 - L - R，高度 = H0 - T

w_raw = self.cfg["D0"] - L - R

h_raw = self.cfg["H0"] - T

w = self.filter(max(0.0, w_raw), self.w_buffer)

h = max(0.0, h_raw)

# 实时信号 (L, R, T, w, h)

self.frame_signal.emit(L, R, T, w, h)

if not self.in_fruit:

if w > self.cfg["enter_threshold"]:

self.enter_cnt += 1

if self.enter_cnt >= self.cfg["enter_count"]:

self.in_fruit = True

self.w_buffer.clear()

self.open_tmp(L, R, T)

self.enter_cnt = 0

else:

self.enter_cnt = 0

else:

self.write_tmp(L, R, T)

if w < self.cfg["exit_threshold"]:

self.exit_cnt += 1

if self.exit_cnt >= self.cfg["exit_count"]:

path = self.close_tmp()

self.process(path)

try:

os.remove(path)

except OSError:

pass

self.in_fruit = False

self.exit_cnt = 0

else:

self.exit_cnt = 0

except Exception as e:

self.status_signal.emit(f"运行异常: {e}")

finally:

if self.ser and self.ser.is_open:

self.ser.close()

self.status_signal.emit("测量已停止")

class ContourDialog(QDialog):

def __init__(self, record, parent=None):

super().__init__(parent)

self.setWindowTitle(f"水果 #{record['id']} 轮廓")

self.resize(600, 400)

layout = QVBoxLayout(self)

self.plot = pg.PlotWidget()

self.plot.setLabel('left', '尺寸', units='mm')

self.plot.setLabel('bottom', '采样点')

self.plot.addLegend()

layout.addWidget(self.plot)

w = record.get('W', [])

h = record.get('H', [])

if w:

self.plot.plot(w, pen=pg.mkPen('b', width=2), name='宽度')

if h:

self.plot.plot(h, pen=pg.mkPen('r', width=2), name='高度')

btn_box = QDialogButtonBox(QDialogButtonBox.Ok)

btn_box.accepted.connect(self.accept)

layout.addWidget(btn_box)

class MainWindow(QMainWindow):

def __init__(self):

super().__init__()

self.setWindowTitle("水果在线测量与分级系统")

self.resize(1200, 700)

central = QWidget()

self.setCentralWidget(central)

main_layout = QHBoxLayout(central)

# 左侧实时曲线

left_widget = QWidget()

left_layout = QVBoxLayout(left_widget)

self.real_time_plot = pg.PlotWidget(title="实时宽度/高度")

self.real_time_plot.setLabel('left', '值', units='mm')

self.real_time_plot.setLabel('bottom', '采样序号')

self.real_time_plot.addLegend()

self.curve_w = self.real_time_plot.plot([], pen=pg.mkPen('b', width=2), name='宽度')

self.curve_h = self.real_time_plot.plot([], pen=pg.mkPen('r', width=2), name='高度')

left_layout.addWidget(self.real_time_plot)

self.buffer_w = []

self.buffer_h = []

# 右侧信息面板

right_widget = QWidget()

right_layout = QVBoxLayout(right_widget)

info_group = QWidget()

info_layout = QGridLayout(info_group)

info_layout.addWidget(QLabel("<b>当前水果信息</b>"), 0, 0, 1, 2)

self.lbl_id = QLabel("ID: --")

self.lbl_w = QLabel("宽度: -- mm")

self.lbl_h = QLabel("高度: -- mm")

self.lbl_len = QLabel("长度: -- mm")

self.lbl_round = QLabel("圆度: --")

self.lbl_grade = QLabel("等级: --")

info_layout.addWidget(self.lbl_id, 1, 0)

info_layout.addWidget(self.lbl_w, 2, 0)

info_layout.addWidget(self.lbl_h, 3, 0)

info_layout.addWidget(self.lbl_len, 1, 1)

info_layout.addWidget(self.lbl_round, 2, 1)

info_layout.addWidget(self.lbl_grade, 3, 1)

right_layout.addWidget(info_group)

history_group = QWidget()

history_layout = QVBoxLayout(history_group)

history_layout.addWidget(QLabel("<b>历史记录</b> (双击查看轮廓)"))

self.table = QTableWidget()

self.table.setColumnCount(7)

self.table.setHorizontalHeaderLabels(["ID", "时间", "宽度(mm)", "高度(mm)", "长度(mm)", "圆度", "等级"])

self.table.horizontalHeader().setSectionResizeMode(QHeaderView.Stretch)

self.table.setEditTriggers(QTableWidget.NoEditTriggers)

self.table.setSelectionBehavior(QTableWidget.SelectRows)

self.table.doubleClicked.connect(self.on_table_double_click)

history_layout.addWidget(self.table)

right_layout.addWidget(history_group)

btn_layout = QHBoxLayout()

self.btn_start = QPushButton("启动测量")

self.btn_stop = QPushButton("停止测量")

self.btn_stop.setEnabled(False)

btn_layout.addWidget(self.btn_start)

btn_layout.addWidget(self.btn_stop)

right_layout.addLayout(btn_layout)

self.status_label = QLabel("就绪")

right_layout.addWidget(self.status_label)

splitter = QSplitter(Qt.Horizontal)

splitter.addWidget(left_widget)

splitter.addWidget(right_widget)

splitter.setSizes([700, 500])

main_layout.addWidget(splitter)

self.status_bar = QStatusBar()

self.setStatusBar(self.status_bar)

self.status_bar.showMessage("系统就绪")

self.btn_start.clicked.connect(self.start_measure)

self.btn_stop.clicked.connect(self.stop_measure)

self.worker_thread = None

self.processor = None

def start_measure(self):

if self.worker_thread and self.worker_thread.isRunning():

return

self.worker_thread = QThread()

self.processor = FruitProcessor(CONFIG)

self.processor.moveToThread(self.worker_thread)

self.processor.frame_signal.connect(self.update_real_time)

self.processor.fruit_done_signal.connect(self.on_fruit_done)

self.processor.status_signal.connect(self.update_status)

self.worker_thread.started.connect(self.processor.run)

self.worker_thread.finished.connect(self.worker_thread.deleteLater)

self.worker_thread.finished.connect(self.on_worker_finished)

self.worker_thread.start()

self.btn_start.setEnabled(False)

self.btn_stop.setEnabled(True)

self.status_bar.showMessage("测量运行中...")

def stop_measure(self):

if self.processor:

self.processor.stop()

self.status_bar.showMessage("正在停止...")

self.btn_stop.setEnabled(False)

def on_worker_finished(self):

self.btn_start.setEnabled(True)

self.btn_stop.setEnabled(False)

self.status_bar.showMessage("测量已停止")

self.processor = None

self.worker_thread = None

def update_real_time(self, L, R, T, w, h):

self.buffer_w.append(w)

self.buffer_h.append(h)

if len(self.buffer_w) > 200:

self.buffer_w.pop(0)

self.buffer_h.pop(0)

self.curve_w.setData(self.buffer_w)

self.curve_h.setData(self.buffer_h)

if self.processor and self.processor.in_fruit:

self.lbl_id.setText(f"ID: {self.processor.fruit_id}")

self.lbl_w.setText(f"宽度: {w:.1f} mm")

self.lbl_h.setText(f"高度: {h:.1f} mm")

else:

if self.processor:

self.lbl_id.setText("ID: --")

def on_fruit_done(self, record):

row = self.table.rowCount()

self.table.insertRow(row)

self.table.setItem(row, 0, QTableWidgetItem(str(record['id'])))

self.table.setItem(row, 1, QTableWidgetItem(record['time']))

self.table.setItem(row, 2, QTableWidgetItem(f"{record['max_w']:.1f}"))

self.table.setItem(row, 3, QTableWidgetItem(f"{record['max_h']:.1f}"))

self.table.setItem(row, 4, QTableWidgetItem(f"{record['length']:.1f}"))

self.table.setItem(row, 5, QTableWidgetItem(f"{record['roundness']:.3f}"))

self.table.setItem(row, 6, QTableWidgetItem(record['grade']))

self.table.scrollToBottom()

self.lbl_w.setText(f"宽度: {record['max_w']:.1f} mm")

self.lbl_h.setText(f"高度: {record['max_h']:.1f} mm")

self.lbl_len.setText(f"长度: {record['length']:.1f} mm")

self.lbl_round.setText(f"圆度: {record['roundness']:.3f}")

self.lbl_grade.setText(f"等级: {record['grade']}")

def update_status(self, msg):

self.status_label.setText(msg)

self.status_bar.showMessage(msg)

def on_table_double_click(self, index):

row = index.row()

if row < 0:

return

id_item = self.table.item(row, 0)

if id_item is None:

return

fruit_id = int(id_item.text())

try:

with open(CONFIG["result_file"], "r", encoding='utf-8') as f:

for line in f:

rec = json.loads(line.strip())

if rec['id'] == fruit_id:

dlg = ContourDialog(rec, self)

dlg.exec_()

return

except Exception as e:

QMessageBox.warning(self, "错误", f"无法加载轮廓数据: {e}")

def closeEvent(self, event):

if self.worker_thread and self.worker_thread.isRunning():

self.stop_measure()

self.worker_thread.quit()

self.worker_thread.wait(2000)

event.accept()

if __name__ == "__main__":

app = QApplication(sys.argv)

win = MainWindow()

win.show()

sys.exit(app.exec_())